This document provides specific assurance requirements for all products purchased by Otto Konigslow Manufacturing Co. as specified on the Purchase Order (PO) of which this is part, to assure that such supplies conform to the required levels of quality and reliability. The quality codes listed below are numbered to correspond to numbers listed on the PO and are applicable to the product or service the seller is to furnish. The seller’s failure to comply with all codes specified on the PO will be cause for rejection of the supplies furnished.

The Standard Requirements (Code 1) apply in their entirety to all PO’s.

1)	Standard Requirements.

- QMS. The Seller shall have a Quality Management System (QMS) equivalent to ISO 9100 / AS9100 and is capable of fulfilling the quality codes as specified on the Otto Konigslow PO.

- Survey / Audit. The Seller’s inspection / QMS shall be subject to survey / audit at the direction of the Otto Konigslow Buyer and / or Otto Konigslow Quality Manager.

- Product Conformity. Unless otherwise specified on the PO, or by written approval, the product or service supplied by the Seller must conform to all drawings, specifications and PO requirements. The Seller shall make its employees aware of their contribution to product conformity.

- Handling of Sub-tiers. The quality requirements specified on the Otto Konigslow PO also applies to the Seller’s sub-tier suppliers. The Seller accepts full responsibility to insure sub-tier supplier compliance with all specified codes and Otto Konigslow and Otto Konigslow’s customer’s requirements.

- Inspection / Nonconformances. All supplies are subject to Otto Konigslow receiving audit / inspection. Nonconforming material will be held, either at Otto Konigslow or at the Seller’s facility until an authorization / disposition is obtained in writing from Otto Konigslow or its customer. All material dispositioned as return to supplier shall be at the Seller’s expense.

- Record Retention. The Seller shall retain all associated records / documentation for 10 yrs. Flight Safety Part records / documentation must be retained for 40 yrs.

- Right of Entry. All work performed shall be subject to inspection, surveillance and / or test by Otto Konigslow, its customer representatives and / or Governmental and Regulatory Agencies at all reasonable times, including the period of performance, at the Seller’s facility or any other entity engaged in the performance of work to fulfill the PO.

- Notification of Changes. The Seller shall notify Otto Konigslow of any changes in the certification / registration / accreditation or major audit findings within (2) business days of receiving notification of the change / finding. Examples of changes in registration include new certification, suspension or expiration. The Seller shall notify Otto Konigslow of changes that may affect quality within (7) days. Examples of changes include ownership, company name change, Management, manufacturing location, process, product and inspection techniques.

- Communicating Language. The Seller shall have the capability to communicate in English. All documentation shall be in English. In cases where the Seller maintains copies in their native language as well as in English and there is a conflict, the English document shall take precedence.

- Competence. The Seller shall determine the necessary competence / qualifications of person(s) doing work under its control that affects the performance and effectiveness of the QMS. Competency shall be based on appropriate education, training or experience.

- Counterfeit Prevention. The Seller shall plan, implement and control processes appropriate to its organization and the product for the prevention of counterfeit or suspect counterfeit part use and their inclusion in product(s) delivered to Otto Konigslow. Product will only be accepted by Otto Konigslow with documentation certifying unbroken traceability (e.g. serialization, lot or batch number) throughout product life.

- Product Safety. The Seller shall have control of the processes needed to assure product safety during the entire product life cycle, as appropriate to its organization and product. The Seller shall raise awareness of product safety to its employees.

- Ethical Behavior. The Seller shall set forth the shared accountability of each of its employees to conduct business with honesty and integrity. Example: Code of Ethics or equivalent.

- Performance Monitoring. Otto Konigslow monitors the Seller’s delivery and quality of products received.  Otto Konigslow may issue a Supplier Corrective Action Request (SCAR) as deemed necessary. The supplier is required to address the root cause(s) and implement corrective action(s) when SCAR’s are issued. To prevent further action, the Seller shall provide an acceptable RCCA by the date requested.

2)	First Article Review / Approval.

The Seller shall submit one part for first article approval. This shall include a complete inspection on one tagged / identified part in the first shipment or, if directed by the buyer, prior to production. A report shall accompany the tagged / identified part listing all drawing requirements including notes and the actual results. The first article part must be a true representative sample and have been produced by the normal manufacturing practices to be used on production parts.

3)	In-Process Controls.

The Seller shall have a system for controlling the hardware throughout the manufacturing process. This may be accomplished using a manufacturing outline, part traveler, shop routing or other suitable method which details the planned step by step operation sequence, inspection points and provisions for controlling the drawing / specification revision status during the manufacturing process.

4)	Material Certification.

The Seller is required to furnish material mill certification, i.e. physical and chemical test reports as applicable in accordance with the specifications specified on this order.

5)	Test Specimens.

The Seller is required to provide test specimens for design approval or inspection / verification purposes.

6)	Otto Konigslow Supplied Material.

The materials furnished on this order have been provided by Otto Konigslow for the specific supplies specified on this order. Traceability must be maintained at all times. No substitution of material composition, size or specification will be allowed unless written authorization is received from Otto Konigslow.

7)	Calibration.

The Seller shall implement and maintain a system for control and use of inspection and / or processing equipment in accordance with MIL-STD-45662. All inspection and / or processing equipment shall be calibrated against certified standards which have known valid relationships traceable to National Standards.

8)	Nonconforming Material.

The Seller shall implement and maintain a system for control of nonconforming material, a system for assuring prompt action to correct the cause of nonconforming material and a system for withholding and segregating nonconforming material from normal production pending disposition.

9)	Final Inspection, Test / Verification.

Prior to shipment to Otto Konigslow, the Seller will perform a final inspection as required to substantiate the purchase order requirements. Documented in process inspection may be used as substantiation for reduced final inspection.

10)	Certification of Conformance.

Each shipment must be accompanied with a signed and dated Certificate of Conformance attesting that supplies or services provided were performed in accordance with all specified purchase order requirements.

11)	Government  / Source Inspection.

The Government, Regulatory Agencies, Otto Konigslow Customers and / or Otto Konigslow reserve the right to inspect, at the seller’s plant, any and all materials specified on this purchase order.

12)	Protection of Materials in Transit.

All material delivered on this order must be packed in a manner to prevent any damage in shipment and storage. All packages are to be properly identified

13)	Inspection Gaging.

The Seller is held responsible for the protection and care, other than normal wear, of all inspection gaging that may be loaned by Otto Konigslow. Loaned gaging, or replacement gaging of equal quality shall be returned in acceptable condition upon completion of this order or upon demand or notice. Unless otherwise specified, said gaging is subject to recall for Otto Konigslow calibration.

14)	Tooling.

Tooling furnished by Otto Konigslow shall be used for production of supplies on this PO. Unauthorized alteration of said tooling is prohibited. Tooling damage while in Seller’s possession will be debited to the Seller’s account. Tooling shall be returned in an acceptable condition upon demand or notice.

15)	Special Process Procedure Approval.

The Seller is required to submit a written procedure, manufacturing outline, part traveler or shop routing detailing the planned step by step operation sequence, inspection points and provisions intended for processing for approval. No operation shall be processed until authorization / approval is secured, in writing, from Otto Konigslow.

16)	Nadcap Certification Required.

This Seller and all members of their supply chain shall be Nadcap certified.

17)	Controlled-Significant Processes.

The process, procedure and / or operation specified on this order is / are under vendor substantiation requirements and must be performed in accordance with a previously approved procedure. No changes to this approved procedure are permitted without the expressed written consent of Otto Konigslow.

18)	Statistical Process Control.

The Seller shall implement and maintain a system for Statistical Process Control (SPC). The Seller’s control plan and procedures shall be submitted to the Otto Konigslow Quality Team for review and approval. All control plans, control charts, capability studies and any corrective actions taken on the process shall be provided to Otto Konigslow with each shipment.

19)	Critical Items / Key Characteristics.

The supplies specified on this order contain product and / or features identified as a “Critical Item” or “Key Characteristic”. When identified as such, additional action(s) from the Seller is required. Special instructions shall be called out on the PO on how they are to be addressed.

20)	Special Packaging Requirements.

The supplies specified on this order require specific packaging to prevent any damage in transit or storage. All special packaging requirements shall be specified on the Otto Konigslow PO or separate document.

21)	Special Handling Requirements.

The supplies and / or products specified on this order are precision hardware with stringent dimensional and cosmetic requirements. Extreme care and handling shall be exercised to prevent any damage. All damage or suspect damage shall be reported immediately to Otto Konigslow.

22)	Mercury Contamination.

All documentation provided by the Seller shall state the following: “EQUIPMENT CONTAINING MERCURY OR MERCURY COMPOUNDS HAS NOT BEEN USED FOR FABRICATION, PACKAGING, EXAMINATION OR TESTING OF PARTS, ASSEMBLIES OR MATERIAL SUPPLIED ON THIS ORDER.”

23)	Calibration Services.

Contractors performing calibration services for Otto Konigslow shall use equipment which has been calibrated and maintained using standards which are traceable to the National Institute of Standards and Technology (NIST).

Calibration certifications for equipment will contain the following information as a minimum:

a)	Supplier name.

b)	Suppliers Certification number.

- c)	Equipment description.

- d)	Equipment ID (model no., serial no.).

- e)	Date calibration performed.

- f)	Due date of next calibration.

- g)	Description of standard(s) and method(s) used for calibration.

- h)	NIST test number of standard(s).

- i)	Date of standard(s) calibration.

- j)	Due date of standard(s) next calibration.

- k)	Identification of personnel performing calibration services.

- l)	Measured values.

- m)	Calibration Procedure used.

- n)	Applicable environment conditions.

Measuring, test and processing equipment shall be packed adequately to prevent damage in shipment or storage. All standards or reference standards which are found to be out of manufacturer’s specified tolerances shall be immediately reported to the Otto Konigslow Quality Team.

24)	Fraud and Falsification.

All Seller documentation, including but not limited to, manufacturing, inspection and test records, shall contain the following preprinted statement: “THE RECORDING OF FALSE, FICTITIOUS OR FRAUDULENT STATEMENTS OR ENTRIES ON THIS DOCUMENT MAY BE PUNISHABLE AS A FELONY UNDER FEDERAL STATUTE.”

(Ref. Federal Law, Title 18, Chapter 47.)

25)	Drawing Control.

The drawing(s) provided are to be considered as customer property and must be returned upon completion of this order.

ITAR WARNING STATEMENT:

The export of documentation or files sent that contain technical data is, or may be, restricted by the Arms Export Control Act and the International Traffic in Arms Regulations (ITAR). Diversion contrary to U.S. law is prohibited. The export, re-export, transfer, or re-transfer of this technical data to any other company, entity, person, or destination, or for any use or purpose other than that for which the technical data was originally provided by the customer is prohibited without prior written approval from the customer and authorization under applicable export control laws.

EAR WARNING STATEMENT:

The export of documentation or files sent that contain technical data is, or may be, restricted by the Arms Export Control Act and the Export Administration Regulations (EAR). Diversion contrary to U.S. law is prohibited. The export, re-export, transfer, or re-transfer of this technical data to any other company, entity, person, or destination, or for any use or purpose other than that for which the technical data was originally provided by the customer is prohibited without prior written approval from the customer and authorization under applicable export control laws.

CUSTOMER SPECIFIC QUALITY CODES

26)

| 148 | PACKING SLIPS |
| --- | --- |
| 176 | THE SHIPPING INSTRUCTIONS AND ROUTING |
| 218 | RATED ORDERS |
| 244 | QUALITY REMARK AEPD |
| 268 | QUALITY REQUIREMENT |
| 357 | QUALITY:DATA IN ENGLISH |
| 379 | LABORATORY TEST REPORTS |
| 398 | DRAWBACK |
| 736 | STOP WORK NOTICE |
| 784 | INDEPENDENT LABORATORY CERTIFICATION |
| B12 | PURCHASE AGREEMENTS |
| C64 | TERMS AND CONDITIONS OF PURCHASE |
| C88 | PO's |
| D75 | GE SUPPLIER RELEASE PLAN (SRP) |
| E09 | SHIPPING INSTRUCTIONS |
| E12 | INSTRUCTIONS FOR PREPARING SHIPMENTS |
| E14 | DPAS RATING - DOA1 |
| F13 | S1002 |
| F47 | GOVERNMENT ACQUISITION OF COMMERCIAL ITEMS |
| G00 | QUALITY ASSURANCE REPRESENTATIVE |
| G54 | SELLER SHALL NOT MAKE |
| G75 | QUALITY: CLASS Y |
| H61 | SMALL BUSINESS PLAN |